| SEND | RECEIVE |
| --- | --- |
| ON BOOTUP (Single Time) /philbrickev/ac/phone/requestid { “request” : 1 } If no response is received then device is not connected to interent Try 2-3 times and show user charger is not responsive | ACK /philbrickev/ac/phone/responseid { “deviceid”: “xyzdfsdfg89234”, “type” : 7/11/22 “version”: 3.2 } |
| PHONE TO EV CHARGER /philbrickev/ac/phone/start { “start” : 1 } Try this frame 5 times or fail if charger does not respond Show error on phone screen | ACK /philbrickev/ac/ev/ack/start { “ack”: “start ok” } |
| PHONE TO EV CHARGER /philbrickev/ac/phone/stop { “stop” : 1 } Try this frame 5 times or fail if charger does not respond Show error on phone screen | ACK /philbrickev/ac/ev/ack/stop { “ack”: “stop ok” } |
| NO ACK Required | EV Charger will send this frame every 3 seconds or on data change /philbrickev/ac/ev/status { “cp”: A/B/C/D/E (Any one ) “voltage”: 230, “current”: 32, “power”: 6.9, “temperature”: 45, “setCurrent” : 16 } |
| ACK /philbrickev/ac/phone/error { “error” : “ok” } | EV Charger will send this frame /philbrickev/ac/ev/error { “error” : 1(Bitwise same as we did in Elevator app) } EV charger will send this frame until it has not received ACK |
| Schedule Charging will happen on mobile app it will just send a start command to EV charger |  |
| Firmware updates requires downloading a bin file and sending it in 1024 packets format. I will explain it later with a live python file which we use to update it. Just copy the same code will work |  |

User should be able to edit Device name as they might have multiple chargers and want to control it